# iSWaMS Receiver Code

#include <SPI.h>

#include <LoRa.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <ArduinoJson.h>

// ==========================================

//             PIN DEFINITIONS

// ==========================================

// LoRa Pins (Your Specific Wiring)

#define SCK_PIN     18

#define MISO_PIN    19

#define MOSI_PIN    23

#define SS_PIN      32

#define RST_PIN     14

#define DIO0_PIN    4

// LCD Settings

#define LCD_ADDR    0x27 // If 0x27 doesn't work, try 0x3F

#define LCD_COLS    16

#define LCD_ROWS    2

LiquidCrystal_I2C lcd(LCD_ADDR, LCD_COLS, LCD_ROWS);

// Onboard LED for Debugging

#define ONBOARD_LED 2

// *** NEW: Buzzer Pin ***

#define BUZZER_PIN  27

// ==========================================

//           GLOBAL DATA STORAGE

// ==========================================

// System Alerts

bool alert_flame = false;

bool alert_temp = false;

bool alert_full = false;

bool alert_tout = false;

bool alert_motion = false;

// *** NEW: RFID Alerts ***

bool alert_entry = false;

bool alert_exit = false;

// GPS Data

String gps_lat = "0.000000";

String gps_lng = "0.000000";

bool gps_connected = false;

// Env Data

String env_temp = "0.0";

String env_hum = "0.0";

// Bin Data

String bin_level = "0.0";

String bin_dist = "0.0";

// RFID Data

String rfid_uid = "None";

int rfid_count = 0;

String rfid_status = "Waiting";

// Display Logic

unsigned long lastDisplaySwitch = 0;

const int displayInterval = 3000;

int currentScreen = 0;

// ==========================================

//               SETUP

// ==========================================

void setup() {

// 1. Initialize Serial, LED, and Buzzer

Serial.begin(115200);

pinMode(ONBOARD_LED, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW); // Ensure buzzer is off initially

// Blink LED 3 times to visually show ESP32 has reset and is running

for(int i=0; i<3; i++) {

digitalWrite(ONBOARD_LED, HIGH); delay(100);

digitalWrite(ONBOARD_LED, LOW);  delay(100);

}

Serial.println("\n\n=== RECEIVER BOOTING ===");

// 2. Init LCD

Wire.begin();

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("System Boot...");

Serial.println("LCD Initialized.");

delay(500);

// 3. Init LoRa

Serial.println("Starting LoRa...");

SPI.begin(SCK_PIN, MISO_PIN, MOSI_PIN, SS_PIN);

LoRa.setSPI(SPI);

LoRa.setPins(SS_PIN, RST_PIN, DIO0_PIN);

if (!LoRa.begin(433E6)) {

Serial.println("❌ LoRa FAILURE!");

Serial.println("Check: 1. Wiring (MISO/MOSI correct?)");

Serial.println("       2. Power (3.3V?)");

lcd.clear();

lcd.print("LoRa Error!");

lcd.setCursor(0,1);

lcd.print("Check Wiring");

while (1) {

// Blink fast to indicate error

digitalWrite(ONBOARD_LED, HIGH); delay(50);

digitalWrite(ONBOARD_LED, LOW);  delay(50);

}

}

LoRa.setSyncWord(0xF3);

Serial.println("✅ LoRa SUCCESS!");

Serial.println("Listening for 0xF3 packets on 433MHz...");

lcd.clear();

lcd.print("LoRa Ready");

lcd.setCursor(0,1);

lcd.print("Waiting...");

}

// ==========================================

//             FUNCTIONS

// ==========================================

void processJson(String jsonString) {

JsonDocument doc;

DeserializationError error = deserializeJson(doc, jsonString);

if (error) {

Serial.print("⚠️ JSON Error: ");

Serial.println(error.c_str());

return;

}

// Alerts - Read from JSON

if(doc.containsKey("alert_flame")) alert_flame = (doc["alert_flame"] == 1);

if(doc.containsKey("alert_temp"))  alert_temp  = (doc["alert_temp"] == 1);

if(doc.containsKey("alert_full"))  alert_full  = (doc["alert_full"] == 1);

if(doc.containsKey("alert_tout"))  alert_tout  = (doc["alert_tout"] == 1);

// *** NEW: Read Entry/Exit Alerts ***

if(doc.containsKey("alert_entry")) alert_entry = (doc["alert_entry"] == 1);

if(doc.containsKey("alert_exit"))  alert_exit  = (doc["alert_exit"] == 1);

// Parsing based on type

const char* type = doc["type"];

Serial.print("Packet Type: "); Serial.println(type);

if (strcmp(type, "GPS") == 0) {

if(doc.containsKey("lat")) gps_lat = doc["lat"].as<String>();

if(doc.containsKey("lng")) gps_lng = doc["lng"].as<String>();

gps_connected = (doc["gps_connected"] == 1);

}

else if (strcmp(type, "ENV") == 0) {

if(doc.containsKey("dht_temp")) env_temp = doc["dht_temp"].as<String>();

if(doc.containsKey("humidity")) env_hum  = doc["humidity"].as<String>();

}

else if (strcmp(type, "BIN") == 0) {

if(doc.containsKey("level")) bin_level = doc["level"].as<String>();

if(doc.containsKey("distance")) bin_dist = doc["distance"].as<String>();

if(doc.containsKey("motion")) alert_motion = (doc["motion"] == 1);

}

else if (strcmp(type, "ID") == 0) {

if(doc.containsKey("uid")) rfid_uid = doc["uid"].as<String>();

if(doc.containsKey("users_count")) rfid_count = doc["users_count"].as<int>();

// Status Logic for LCD Text

int isEntry = doc["entry"];

int isExit = doc["exit"];

if (isEntry == 1) rfid_status = "ENTRY";

else if (isExit == 1) rfid_status = "EXIT";

else rfid_status = "Logged";

}

}

void showEnvironment() {

lcd.setCursor(0, 0);

lcd.print("Temp: "); lcd.print(env_temp); lcd.print("C   ");

lcd.setCursor(0, 1);

lcd.print("Hum : "); lcd.print(env_hum); lcd.print("%   ");

}

void showBinStatus() {

lcd.setCursor(0, 0);

lcd.print("Bin Lvl: "); lcd.print(bin_level); lcd.print("%   ");

lcd.setCursor(0, 1);

lcd.print(alert_motion ? "Motion: YES     " : "Motion: NO      ");

}

void showGPS() {

lcd.setCursor(0, 0);

if (!gps_connected) {

lcd.print("GPS: No Fix     ");

lcd.setCursor(0, 1);

lcd.print("Searching...    ");

} else {

lcd.print("L:"); lcd.print(gps_lat);

lcd.setCursor(0, 1);

lcd.print("N:"); lcd.print(gps_lng);

}

}

void showRFID() {

lcd.setCursor(0, 0);

lcd.print("ID:"); lcd.print(rfid_uid);

for(int i=3+rfid_uid.length(); i<16; i++) lcd.print(" ");

lcd.setCursor(0, 1);

lcd.print(rfid_status);

lcd.print(" (Cnt:"); lcd.print(rfid_count); lcd.print(")");

}

void showAlert(String line1, String line2) {

lcd.setCursor(0, 0);

lcd.print(line1);

for(int i=line1.length(); i<16; i++) lcd.print(" ");

lcd.setCursor(0, 1);

lcd.print(line2);

for(int i=line2.length(); i<16; i++) lcd.print(" ");

}

// ==========================================

//               LOOP

// ==========================================

void loop() {

// 1. Check for LoRa Packets

int packetSize = LoRa.parsePacket();

if (packetSize) {

// Blink LED briefly on receive

digitalWrite(ONBOARD_LED, HIGH);

String recv = "";

while (LoRa.available()) {

recv += (char)LoRa.read();

}

digitalWrite(ONBOARD_LED, LOW);

Serial.print("RX ("); Serial.print(packetSize); Serial.print(" bytes): ");

Serial.println(recv);

processJson(recv);

}

// 2. LCD Management & BUZZER LOGIC

unsigned long now = millis();

// --- CRITICAL ALERT BUZZER LOGIC ---

// Buzzer rings if ANY critical condition is true

if (alert_flame || alert_full || alert_tout || alert_temp || alert_entry || alert_exit) {

digitalWrite(BUZZER_PIN, HIGH);

} else {

digitalWrite(BUZZER_PIN, LOW);

}

// --- CRITICAL ALERT SCREEN OVERRIDE ---

if (alert_flame) { showAlert("!!! FIRE !!!", "DETECTED"); return; }

if (alert_full)  { showAlert("!!! ALERT !!!", "BIN IS FULL"); return; }

if (alert_tout)  { showAlert("!!! TIMEOUT !!!", "RFID RESET"); return; }

if (alert_temp)  { showAlert("!!! HEAT !!!", "HIGH TEMP"); return; }

if (alert_entry) { showAlert("RFID ALERT:", "USER ENTRY"); return; }

if (alert_exit)  { showAlert("RFID ALERT:", "USER EXIT"); return; }

// --- NORMAL CYCLIC DISPLAY ---

if (now - lastDisplaySwitch > displayInterval) {

lastDisplaySwitch = now;

lcd.clear();

currentScreen++;

if (currentScreen > 3) currentScreen = 0;

switch (currentScreen) {

case 0: showEnvironment(); break;

case 1: showBinStatus(); break;

case 2: showGPS(); break;

case 3: showRFID(); break;

}

}

}