# iSWaMS Transmitter Code

#include <WiFi.h>                        // Library for ESP-32 WiFi

#include <PubSubClient.h>                // library for MQTT publishing

#include <Wire.h>                        // library for I2C

#include <DHT.h>                         // library for DHT22

#include <TinyGPS++.h>                   // library for NEO-6M GPS

#include <Adafruit_PWMServoDriver.h>     // library for PCA9685 Servo Driver

#include <SPI.h>                         // Shared SPI bus library for RFID-RC522 & LoRa Ra-02

#include <MFRC522.h>                     // library for RFID-RC522

#include <LoRa.h>                        // library for LoRa Ra-02

#include <ArduinoJson.h>                 // library for Transmitting ArduinoJson Payload (used instead of string concatenation)

// ==========================================

//              PIN DEFINITIONS

// ==========================================

// -------- SHARED SPI BUS (LoRa + RFID) --------

#define SPI_SCK     18

#define SPI_MISO    19

#define SPI_MOSI    23

// -------- RFID RC522 Pins --------

#define RFID_SDA     5

#define RFID_RST    33

// -------- LoRa Ra-02 Pins --------

#define LORA_NSS    32

#define LORA_RST    14

#define LORA_DIO0    4

// -------- HC-SR04 SENSOR --------

#define TRIG_PIN    25

#define ECHO_PIN    34

//-------- LED --------

#define LED_PIN      2

//-------- PIR SENSOR --------

#define PIR_PIN     27

//-------- DHT22 SENSOR --------

#define DHT_PIN     26

#define DHT_TYPE    DHT22

// -------- I2C & GPS --------

#define I2C_SDA     21

#define I2C_SCL     22

#define GPS_RX      16

#define GPS_TX      17

// -------- FLAME SENSOR --------

#define FLAME_AO    15   // Analog output (intensity)

#define FLAME_DO    13   // Digital output (LOW = flame detected)

// ==========================================

//              GLOBAL OBJECTS

// ==========================================

// Bin Calibration

#define MAX_DISTANCE 16.0

#define MIN_DISTANCE 3.0

#define FULL_LEVEL 90

// *** SERVO CONFIGURATION ***

#define SERVO_MIN 150  // Min pulse length (approx 0 degrees)

#define SERVO_MAX 600  // Max pulse length (approx 180 degrees)

#define SERVO_PORT_1 11

#define SERVO_PORT_2 15

// WiFi & MQTT

const char* ssid = "Batman_ACT";

const char* password = "65716553";

const char* mqtt_server = "thingsboard.cloud";

const int mqtt_port = 1883;

const char* access_token = "es57KQNLXHyy0DDrhvri";

WiFiClient espClient;

PubSubClient client(espClient);

// Sensors

DHT dht(DHT_PIN, DHT_TYPE);

TinyGPSPlus gps;

HardwareSerial GPS_Serial(2);

MFRC522 rfid(RFID_SDA, RFID_RST);

Adafruit_PWMServoDriver pca9685 = Adafruit_PWMServoDriver();

// Logic Variables

bool pcaConnected = false;

bool gpsSerialStarted = false;

bool gpsConnected = false;

unsigned long lastGPSdata = 0;

unsigned long lastCalibrationTime = 0;

const unsigned long calibrationInterval = 3600000UL;

// LoRa Logic

bool loraOK = false;

unsigned long lastLoRaRetry = 0;

const unsigned long loRaRetryInterval = 30000UL;

unsigned long lastLoRaTime = 0;

// *** NEW: LoRa Sequence Counter for Cyclic Transmission ***

int loraSequence = 0;

// 0=GPS, 1=Environment(DHT/Flame), 2=Bin(Ultra/PIR), 3=RFID, 4=System

// RFID Logic

String lastRfidUID = "None";

unsigned long lastRfidTime = 0;

unsigned long lastTelemetryTime = 0;

// ===== RFID multi-user table + timeout (Case B: global reset) =====

const int MAX_USERS = 20;

struct RfidUser {

String uid;

uint8_t state; // 0 = NONE, 1 = ENTRY, 2 = EXIT

unsigned long lastSeen;

};

RfidUser users[MAX_USERS];

int userCount = 0;

unsigned long lastRFIDActivity = 0; // last time any RFID tap happened

const unsigned long rfidGlobalTimeout = 60000UL; // 1 minute

bool rfidTimeoutAlertSent = false; // to avoid repeating timeout alert until activity resumes

// Telemetry alert flags (kept locally to set when needed)

bool alert_rfid_timeout = false;

// Initialization reporting

bool initPayloadSent = false;

bool init_wifi = false;

bool init_mqtt = false;

bool init_pca = false;

bool init_rfid_flag = false;

bool init_lora = false;

bool init_gps = false;

bool init_dht = false;

bool init_ultrasonic = false;

bool init_pir = false;

bool init_flame = false;

// ==========================================

//              FUNCTIONS

// ==========================================

void setupWiFi() {

Serial.print("Connecting to WiFi: ");

Serial.println(ssid);

WiFi.begin(ssid, password);

unsigned long start = millis();

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

if (millis() - start > 20000) {

Serial.println("\nWarning: WiFi connection timeout");

break;

}

}

init_wifi = (WiFi.status() == WL_CONNECTED);

if (init_wifi) {

Serial.println("\nWiFi Connected!");

Serial.print("IP: "); Serial.println(WiFi.localIP());

} else {

Serial.println("\nWiFi not connected at setup end.");

}

}

void reconnectMQTT() {

if (WiFi.status() != WL_CONNECTED) return;

if (!client.connected()) {

Serial.print("Connecting MQTT...");

String clientId = "ESP32_Bin_" + String(random(0xffff), HEX);

// ThingsBoard: clientId can be anything, access_token used as username

if (client.connect(clientId.c_str(), access_token, NULL)) {

Serial.println("Connected to MQTT!");

init_mqtt = true;

} else {

Serial.print("Failed rc="); Serial.println(client.state());

init_mqtt = false;

}

} else {

init_mqtt = true;

}

}

// -------- Helpers --------

float getDistanceCM() {

digitalWrite(TRIG_PIN, LOW); delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH); delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

long duration = pulseIn(ECHO_PIN, HIGH, 30000);

if (duration == 0) return MAX_DISTANCE + 1;

return (duration * 0.0343) / 2.0;

}

float getLevelPercent(float distance) {

if (distance > MAX_DISTANCE) distance = MAX_DISTANCE;

if (distance < MIN_DISTANCE) distance = MIN_DISTANCE;

return ((MAX_DISTANCE - distance) / (MAX_DISTANCE - MIN_DISTANCE)) * 100.0;

}

// -------- UPDATED GPS READ FUNCTION --------

void readGPS() {

// Continuously feed TinyGPS++ with incoming bytes

while (GPS_Serial.available() > 0) {

gps.encode(GPS_Serial.read());

}

// Mark GPS as "connected" only when we have a valid location fix

gpsConnected = gps.location.isValid();

}

// -------- Servo Control Function --------

// open = true (Ending Position), open = false (Starting Position)

void setServos(bool open) {

int pulse = open ? 2400 : 600;   // MG90 typical

pca9685.writeMicroseconds(SERVO_PORT_1, pulse);

pca9685.writeMicroseconds(SERVO_PORT_1, pulse);

Serial.print("⚙️ Servos moved to: ");

Serial.println(open ? "OPEN (Ending Pos)" : "CLOSED (Starting Pos)");

}

// -------- Hardware Initialization (Shared SPI) --------

void initRFID() {

digitalWrite(LORA_NSS, HIGH);

rfid.PCD_Init();

rfid.PCD_SetAntennaGain(rfid.RxGain_max);

Serial.println("RC522 initialized");

init_rfid_flag = true;

}

// Initialize LoRa

bool tryInitLoRa() {

Serial.print("Initializing LoRa ");

digitalWrite(RFID_SDA, HIGH); // Deselect RFID

// Prepare LoRa Pins

pinMode(LORA_NSS, OUTPUT);

pinMode(LORA_RST, OUTPUT);

digitalWrite(LORA_NSS, HIGH);

// Hard Reset

digitalWrite(LORA_RST, HIGH); delay(100);

digitalWrite(LORA_RST, LOW);  delay(100);

digitalWrite(LORA_RST, HIGH); delay(200);

// Assign SPI

LoRa.setSPI(SPI);

LoRa.setPins(LORA_NSS, LORA_RST, LORA_DIO0);

LoRa.setSyncWord(0xF3);  // ensure sync word is set before begin

if (LoRa.begin(433E6)) {

Serial.println("Success!");

// Set LoRa TX Power (Recommended 18 for MB102 stability)

LoRa.setTxPower(18);

return true;

}

Serial.println("Failed.");

return false;

}

// ===== RFID user helpers =====

int findUserIndex(const String &uid) {

for (int i = 0; i < userCount; i++) {

if (users[i].uid == uid) return i;

}

return -1;

}

int addUser(const String &uid) {

if (userCount >= MAX_USERS) return -1;

users[userCount].uid = uid;

users[userCount].state = 0; // NONE by default

users[userCount].lastSeen = millis();

userCount++;

return userCount - 1;

}

void clearAllUsers() {

for (int i = 0; i < userCount; i++) {

users[i].uid = "";

users[i].state = 0;

users[i].lastSeen = 0;

}

userCount = 0;

}

// Read RFID Tag

void readRFID() {

// Ensure LoRa is quiet

digitalWrite(LORA_NSS, HIGH);

if (!rfid.PICC_IsNewCardPresent()) return;

if (!rfid.PICC_ReadCardSerial()) return;

String rfidStr = "";

for (byte i = 0; i < rfid.uid.size; i++) {

if (rfid.uid.uidByte[i] < 0x10) rfidStr += "0";

rfidStr += String(rfid.uid.uidByte[i], HEX);

}

rfidStr.toUpperCase();

lastRfidUID = rfidStr;

lastRfidTime = millis();

lastRFIDActivity = millis();        // update global last activity

rfidTimeoutAlertSent = false;       // activity resumed -> clear timeout-sent flag

alert_rfid_timeout = false;         // clear the alert flag when activity happens

// Find or add user

int idx = findUserIndex(rfidStr);

if (idx == -1) {

idx = addUser(rfidStr);

Serial.print("New RFID user added idx="); Serial.print(idx);

Serial.print(" UID="); Serial.println(rfidStr);

}

if (idx >= 0) {

// toggle: if NONE or EXIT -> ENTRY, if ENTRY -> EXIT

uint8_t newState;

if (users[idx].state == 1) newState = 2; // ENTRY -> EXIT

else newState = 1; // NONE or EXIT -> ENTRY

users[idx].state = newState;

users[idx].lastSeen = millis();

Serial.print("🏷️ RFID Tag: "); Serial.print(rfidStr);

Serial.print(" -> ");

Serial.println((newState == 1) ? "ENTRY" : "EXIT");

}

// --- SERVO LOGIC ---

// If ANY user is currently in ENTRY state, keep servos OPEN.

// If ALL users have exited (or list is empty), CLOSE servos.

bool anyoneInside = false;

for(int i = 0; i < userCount; i++) {

if(users[i].state == 1) {

anyoneInside = true;

break;

}

}

if (anyoneInside) {

setServos(true); // Move to Ending Position (Open)

} else {

setServos(false); // Move to Starting Position (Closed)

}

rfid.PICC_HaltA();

rfid.PCD_StopCrypto1();

}

// Send initialization JSON to ThingsBoard (once)

void publishInitStatus() {

// Construct JSON using ArduinoJson

JsonDocument doc;

doc["init_wifi"] = init_wifi ? 1 : 0;

doc["init_mqtt"] = init_mqtt ? 1 : 0;

doc["init_pca"] = init_pca ? 1 : 0;

doc["init_rfid"] = init_rfid_flag ? 1 : 0;

doc["init_lora"] = init_lora ? 1 : 0;

doc["init_gps"] = init_gps ? 1 : 0;

doc["init_dht"] = init_dht ? 1 : 0;

doc["init_ultrasonic"] = init_ultrasonic ? 1 : 0;

doc["init_pir"] = init_pir ? 1 : 0;

doc["init_flame"] = init_flame ? 1 : 0;

char payloadBuffer[512];

serializeJson(doc, payloadBuffer);

if (client.connected()) {

client.publish("v1/devices/me/telemetry", payloadBuffer);

Serial.print("☁️ Init payload sent to MQTT: ");

Serial.println(payloadBuffer);

initPayloadSent = true;

} else {

Serial.println("⚠️ MQTT not connected yet; will send init payload when connected");

}

}

// ==========================================

//              SETUP

// ==========================================

void setup() {

Serial.begin(115200);

delay(500);

Serial.println("\n\n===== SYSTEM BOOT =====");

// Pin Modes

pinMode(TRIG_PIN, OUTPUT); pinMode(ECHO_PIN, INPUT);

pinMode(LED_PIN, OUTPUT);  digitalWrite(LED_PIN, LOW);

pinMode(PIR_PIN, INPUT);

pinMode(FLAME_DO, INPUT_PULLUP); // DO is active LOW for flame

// FLAME_AO is analog input - no pinMode required

analogReadResolution(12);

analogSetAttenuation(ADC_11db);

// I2C / PCA9685 detection (serial friendly)

Serial.println("====================================");

Serial.println("🔍 Checking PCA9685 connection...");

Serial.print("🔧 Using I2C Pins -> SDA: ");

Serial.print(I2C_SDA);

Serial.print(" | SCL: ");

Serial.println(I2C_SCL);

Wire.begin(I2C_SDA, I2C_SCL);

Serial.println("📡 Scanning I2C address 0x40...");

Wire.beginTransmission(0x40);

uint8_t error = Wire.endTransmission();

if (error == 0) {

pcaConnected = true;

init_pca = true;

Serial.println("✅ PCA9685 FOUND at address 0x40!");

Serial.println("⚙️ Initializing PCA9685...");

pca9685.begin();

pca9685.setPWMFreq(50);

// Initialize Servos to Closed/Starting Position

setServos(false);

Serial.println("🎉 PCA9685 Initialized Successfully!");

}

else if (error == 4) {

pcaConnected = false;

init_pca = false;

Serial.println("❌ Unknown I2C error while detecting PCA9685!");

}

else {

pcaConnected = false;

init_pca = false;

Serial.println("⚠️ PCA9685 NOT FOUND at address 0x40!");

Serial.print("I2C Error Code: ");

Serial.println(error);

}

Serial.println("====================================");

// Sensors

Serial.println("🔧 Initializing sensors...");

// DHT

dht.begin();

// Try a quick read to confirm DHT presence (may return NaN on some DHT modules immediately)

float t_check = dht.readTemperature();

float h_check = dht.readHumidity();

if (!isnan(t_check) || !isnan(h_check)) {

init_dht = true;

Serial.println("✅ DHT22 appears responsive (quick check succeeded).");

} else {

init_dht = true; // still mark true: we consider DHT wired even if first read fails; it may need time

Serial.println("⚠️ DHT22 initial read returned NaN — it may still work. Marking as present.");

}

// Flame quick check (analog + digital)

int flameAnalogCheck = analogRead(FLAME_AO);

int flameDigitalCheck = digitalRead(FLAME_DO);

// We mark flame as present if AO returns a plausible reading or DO is HIGH/LOW

// Note: analogRead can be 0 even in dark — presence check is simple

init_flame = true; // we assume wired; report actual readings below

Serial.print("🔥 Flame sensor initial analog reading (AO="); Serial.print(FLAME_AO); Serial.print("): ");

Serial.println(flameAnalogCheck);

Serial.print("🔥 Flame sensor initial digital reading (DO="); Serial.print(FLAME_DO); Serial.print(") [LOW = flame]: ");

Serial.println(flameDigitalCheck == LOW ? "LOW (flame)" : "HIGH (no flame)");

// GPS (UART start)

GPS_Serial.begin(9600, SERIAL_8N1, GPS_RX, GPS_TX);

gpsSerialStarted = true;

init_gps = true;

Serial.print("✅ GPS serial started on RX=");

Serial.print(GPS_RX);

Serial.print(" TX=");

Serial.println(GPS_TX);

// Ultrasonic & PIR basic init flag

init_ultrasonic = true;

init_pir = true;

Serial.println("✅ Ultrasonic (HC-SR04) pin and PIR pin configured.");

// WiFi & MQTT setup

setupWiFi();

client.setServer(mqtt_server, mqtt_port);

// Increase buffer size for large JSON payloads

client.setBufferSize(1024);

// try connecting to MQTT immediately (if WiFi OK)

reconnectMQTT();

// -------- SHARED SPI INIT --------

// 1. Manually Deselect both chips immediately to prevent conflicts

pinMode(LORA_NSS, OUTPUT); digitalWrite(LORA_NSS, HIGH);

pinMode(RFID_SDA, OUTPUT); digitalWrite(RFID_SDA, HIGH);

// 2. Start SPI Bus ONCE (with your chosen pins)

SPI.begin(SPI_SCK, SPI_MISO, SPI_MOSI);

// 3. Init RFID

Serial.println("Initializing RFID...");

initRFID();

Serial.println("✅ RFID Ready");

// 4. Init LoRa

loraOK = tryInitLoRa();

init_lora = loraOK;

if (!loraOK) lastLoRaRetry = millis();

// Initialize RFID user table empty

clearAllUsers();

lastRFIDActivity = millis();

// Print a neat hardware init summary to Serial

Serial.println("\n--- INITIALIZATION SUMMARY ---");

Serial.print("WiFi: "); Serial.println(init_wifi ? "OK" : "NOT CONNECTED");

Serial.print("MQTT: "); Serial.println(init_mqtt ? "OK" : "NOT CONNECTED");

Serial.print("PCA9685: "); Serial.println(init_pca ? "FOUND" : "NOT FOUND");

Serial.print("RFID RC522: "); Serial.println(init_rfid_flag ? "INITIALIZED" : "NOT INITIALIZED");

Serial.print("LoRa: "); Serial.println(init_lora ? "INITIALIZED" : "NOT INITIALIZED");

Serial.print("GPS UART: "); Serial.println(init_gps ? "STARTED" : "NOT STARTED");

Serial.print("DHT22: "); Serial.println(init_dht ? "PRESENT" : "NOT PRESENT");

Serial.print("Flame Sensor (AO/DO): "); Serial.println(init_flame ? "PRESENT" : "NOT PRESENT");

Serial.print("HC-SR04: "); Serial.println(init_ultrasonic ? "PIN OK" : "PIN NOT OK");

Serial.print("PIR: "); Serial.println(init_pir ? "PIN OK" : "PIN NOT OK");

Serial.println("-------------------------------");

// If MQTT is connected now, send init status. Otherwise it will be sent from loop when MQTT becomes available.

if (client.connected() && !initPayloadSent) {

publishInitStatus();

} else {

Serial.println("Init payload will be sent once MQTT connects.");

}

}

// ==========================================

//              LOOP

// ==========================================

void loop() {

unsigned long now = millis();

// 1. Maintenance (no flame calibration)

if (!loraOK && (now - lastLoRaRetry >= loRaRetryInterval)) {

loraOK = tryInitLoRa();

initRFID();

lastLoRaRetry = now;

init_lora = loraOK;

}

// Check global RFID inactivity timeout (Case B)

if ((now - lastRFIDActivity) >= rfidGlobalTimeout && !rfidTimeoutAlertSent) {

// If there were any users, clear them and set an alert

if (userCount > 0) {

Serial.println("⏱️ RFID inactivity timeout - clearing all user states");

clearAllUsers();

// *** TIMEOUT OCCURRED: CLOSE SERVOS ***

setServos(false);

alert_rfid_timeout = true;

rfidTimeoutAlertSent = true;

}

}

// MQTT maintenance & ensure initial payload is sent once

if (!client.connected()) reconnectMQTT();

client.loop();

if (client.connected() && !initPayloadSent) {

// update init_mqtt true and send init payload

init_mqtt = true;

publishInitStatus();

}

// 2. Fast Reads

readGPS();

readRFID();

// 3. Telemetry (Every 5 seconds)

if (now - lastTelemetryTime >= 5000) {

lastTelemetryTime = now;

// Gather Data

float dist = getDistanceCM();

float level = getLevelPercent(dist);

int motion = digitalRead(PIR_PIN);

float hum = dht.readHumidity();

float temp = dht.readTemperature();

if (isnan(hum)) hum = 0;

if (isnan(temp)) temp = 0;

// Flame readings

int flameAnalog = analogRead(FLAME_AO);     // 0..4095 (12-bit)

int flameDigital = digitalRead(FLAME_DO);   // LOW = flame detected

bool a_flame = (flameDigital == LOW);

double lat = gps.location.isValid() ? gps.location.lat() : 0.0;

double lng = gps.location.isValid() ? gps.location.lng() : 0.0;

// Alerts

bool a_motion = (motion == HIGH);

bool a_temp = (temp > 45);

bool a_rfid = (now - lastRfidTime < 5000);

bool a_bin_full = (level >= FULL_LEVEL);

if(a_motion) Serial.println("🚨 MOTION DETECTED");

if(a_temp)   Serial.println("🌡️ HIGH TEMPERATURE ALERT!");

if(a_rfid)   Serial.println("🆔 RFID SCANNED");

if(a_bin_full) Serial.println("📦 BIN FULL");

if(a_flame) Serial.println("🔥 FLAME DETECTED");

if(alert_rfid_timeout) Serial.println("⚠️ RFID TIMEOUT ALERT (global reset)");

// --- Determine Entry/Exit Strings ---

String lastUserMode = "NONE";

if (lastRfidUID != "None") {

int idx = findUserIndex(lastRfidUID);

if (idx >= 0) lastUserMode = (users[idx].state == 1) ? "ENTRY" : (users[idx].state == 2) ? "EXIT" : "NONE";

}

// ===============================================

//              MQTT SEND (FULL DATA)

// ===============================================

JsonDocument doc;

doc["distance"]             = serialized(String(dist, 1));

doc["level"]                = serialized(String(level, 1));

doc["motion"]               = a_motion ? 1 : 0;

doc["dht_temp"]             = serialized(String(temp, 1));

doc["humidity"]             = serialized(String(hum, 1));

doc["gps_connected"]        = gpsConnected ? 1 : 0;

doc["lat"]                  = serialized(String(lat, 6));

doc["lng"]                  = serialized(String(lng, 6));

doc["rfid_uid"]             = lastRfidUID;

doc["flame_analog"]         = flameAnalog;

doc["flame_digital"]        = (flameDigital == LOW ? 1 : 0);

doc["alert_flame"]          = a_flame ? 1 : 0;

doc["pca_status"]           = pcaConnected ? 1 : 0;

doc["rfid_users_count"]     = userCount;

doc["rfid_user_state"]      = lastUserMode;

doc["alert_motion"]         = a_motion ? 1 : 0;

doc["alert_temperature"]    = a_temp ? 1 : 0;

doc["alert_rfid"]           = a_rfid ? 1 : 0;

doc["alert_rfid_timeout"]   = alert_rfid_timeout ? 1 : 0;

doc["alert_bin_full"]       = a_bin_full ? 1 : 0;

// --- SEND MQTT ---

if (WiFi.status() == WL_CONNECTED && client.connected()) {

char mqttBuffer[1024];

serializeJson(doc, mqttBuffer);

if(client.publish("v1/devices/me/telemetry", mqttBuffer)) {

Serial.print("☁️ MQTT Sent: ");

Serial.println(mqttBuffer);

} else {

Serial.println("⚠️ MQTT Publish Failed (Payload too big?)");

}

}

// ===============================================

//      LORA SEND (CYCLIC + GLOBAL ALERTS)

// ===============================================

if (loraOK) {

JsonDocument loraDoc;

String logType = "";

// 1. GLOBAL ALERTS (Included in EVERY packet, including RFID Entry/Exit)

loraDoc["alert_temp"]  = a_temp ? 1 : 0;

loraDoc["alert_flame"] = a_flame ? 1 : 0;

loraDoc["alert_full"]  = a_bin_full ? 1 : 0;

loraDoc["alert_tout"]  = alert_rfid_timeout ? 1 : 0;

loraDoc["alert_entry"] = (lastUserMode == "ENTRY" && a_rfid) ? 1 : 0;

loraDoc["alert_exit"]  = (lastUserMode == "EXIT" && a_rfid) ? 1 : 0;

// 2. SPLIT DATA PACKETS

switch (loraSequence) {

case 0: // GPS

loraDoc["type"] = "GPS";

loraDoc["gps_connected"] = gpsConnected ? 1 : 0;

loraDoc["lat"] = serialized(String(lat, 6));

loraDoc["lng"] = serialized(String(lng, 6));

logType = "GPS Data";

break;

case 1: // Environment

loraDoc["type"] = "ENV";

loraDoc["dht_temp"] = serialized(String(temp, 1));

loraDoc["humidity"] = serialized(String(hum, 1));

loraDoc["flame_A"] = flameAnalog;

loraDoc["flame_D"] = (flameDigital == LOW ? 1 : 0);

logType = "Environment (DHT/Flame)";

break;

case 2: // Bin Status

loraDoc["type"] = "BIN";

loraDoc["distance"] = serialized(String(dist, 1));

loraDoc["level"] = serialized(String(level, 1));

loraDoc["motion"] = a_motion ? 1 : 0;

logType = "Bin Sensors";

break;

case 3: // RFID

loraDoc["type"] = "ID";

loraDoc["uid"] = lastRfidUID;

loraDoc["users_count"] = userCount;

// Specific data fields still here for detail

loraDoc["entry"] = (lastUserMode == "ENTRY" && a_rfid) ? 1 : 0;

loraDoc["exit"]  = (lastUserMode == "EXIT" && a_rfid) ? 1 : 0;

logType = "RFID Status";

break;

case 4: // System

loraDoc["type"] = "SYS";

loraDoc["pca"] = pcaConnected ? 1 : 0;

logType = "System Info";

break;

}

// Serialize and Send

char loraBuffer[255];

serializeJson(loraDoc, loraBuffer);

// Ensure shared SPI is in safe state before LoRa TX

digitalWrite(RFID_SDA, HIGH);   // deselect RFID

digitalWrite(LORA_NSS, HIGH);   // idle LoRa CS high before beginPacket()

LoRa.beginPacket();

LoRa.print(loraBuffer);

if(LoRa.endPacket()) {

Serial.print("📡 LoRa Sent (Seq ");

Serial.print(loraSequence);

Serial.print(" - ");

Serial.print(logType);

Serial.print("): ");

Serial.println(loraBuffer);

} else {

Serial.println("⚠️ LoRa Failed");

loraOK = false; lastLoRaRetry = now;

}

// Move to next sensor type for next 5-second interval

loraSequence++;

if (loraSequence > 4) loraSequence = 0;

}

// LED Control

digitalWrite(LED_PIN, (a_bin_full) ? HIGH : LOW);

// after having sent timeout alert once, clear alert flag so telemetry doesn't keep reporting it forever

if (alert_rfid_timeout && rfidTimeoutAlertSent) {

// already reported, clear it to avoid repeated flag in later telemetry

alert_rfid_timeout = false;

}

}

}